## Future new CEA-CNRS white beam microdiffraction beamline at ESRF.

S. Maîtrejean(a), O. Robach(b), O. Ulrich(b), X. Biquard(b), G. Rolland(a), F. Rieutord(b)

(a) DRT/LETI, (b) DRFMC,

CEA-Grenoble ,17 rue des Martyrs, F-38054 GRENOBLE Cedex 9, FRANCE

Contact : S. Maîtrejean, tel : (+33) (0) 4 38 78 97 49, fax : (+33) (0) 4 38 78 30 34

e-mail : smaitrejean@cea.fr

The design of the future CEA-CNRS BM32 white beam microdiffraction beamline at ESRF is presented. This beamline is planned to be the first setup of this type operational in Europe and will be open to international external users. The beamline will cover an energy range of 5 to 40 KeV, with an ultimate predicted beam size of 0.5 x 0.1 µm2. Beam stability will be ensured by using a micro-slit as a secondary x-ray source. Switching between white and monochromatic beam will be realized by inserting a monochromator.

The beamline will allow 2D mapping of the sample microstructure by taking Laue patterns at each x,y position of the sample. For each grain, the crystallographic orientation and the deviatoric strain tensor can be determined. Complementary monochromatic beam measurements then allow to determine the other components of the tensor (dilatational).

Applications of the technique to numerous microelectronics-related problems are foreseen, in particular the correlations between grain structure, line width, and electromigration reliability in interconnects lines.

Preliminary feasibility experiments on the microdiffraction beamline [1] at ALS in Berkeley were performed on Damascene copper interconnects fabricated by the CEA / LETI. A comparison will be presented of the grain structure obtained for different natures of the barrier layer (TaN/Ta or TiN) and different deposition methods for the copper seed layer.

[1] 'Submicron X-ray Diffraction' A.A.MacDowell, R.S.Celestre, N.Tamura, R.Spolenak, B.C. Valek, W.L.Brown, J.C.Bravman, H.A.Padmore, B.W.Batterman & J.R.Patel, Nuclear Instruments and Methods in Physics Research A 467-468 (2001) 936-943